Ainol Novo Mars, Tornados and Aurora II Recovery Kit Instructions

Translated by ZOMBiE CYGIG

Creating the Recovery Image microSD Card

- Download and decompress the relevant .img image file.

- Launch USB Image Tool.

- Insert a blank microSD card to your PC via USB card reader and select it as shown below:

- Select Favorites, and then Add.

- In the pop-up window, browse to and select the .img image file.

- You may rename the image file for convenience, as shown below.

- Click Restore.

- Select Yes to continue upon prompting.

- The program will now flash the image file to your microSD card. This process might take a few minutes.

- The image flashing is now done and the microSD card will appear to be a 512MB card instead of the original capacity.

- You should see the following files, although the file names may vary.

- The ROM in the recovery image has minimal functions and apps, should you wish to slipstream other apps, add them to the root directory of the microSD card.

- USB Image Tool supports Windows XP, Vista and 7, 32 and 64 bits. Microsoft.NET framework is needed for the program, or else a 0xc0000135 error will show up on Windows XP.

Using the Recovery Image microSD Card

1. Shutdown the tablet (For those unsure, unplug charging and USB cable and hold the power button for 10 seconds to force a shutdown).

2. Insert the above prepared microSD card.

3. Holding the Volume Minus button, hold the power button for five seconds, the screen should power up with a logo or the backlight will be lit.

4. The recovery process is automated, after recovery, the tablet will shutdown and reboot.

Restoring the Recovery Image microSD Card

The microSD card will appear as a 512MB card instead of its original capacity. You will only be able to utilize 512MB on the card. USBoot 1.7 can revert your microSD card back to the original capacity.

- Insert the above mentioned Recovery Image microSD Card into your PC using a card reader. Launch USBoot 1.7 with administrator rights.

- Click Yes.

- Select your microSD card.

- Click on “点击此处选择工作模式” at the bottom of the window, right above FOROO@TOM.COM. Choose the option as shown in the image below:

- Click on the Start button next to the red spot as shown below:

- When the message in blue shows up, it means that the processing is done. Safely eject and plug in your card reader (with your microSD card still inside of course) and your microSD card should now reflect its original capacity.